PICKUP TRUCK, 4 DOOR

THE TRUCK SHALL BE PROVIDED WITH ALL NEW PARTS, THE MANUFACTURER’S STANDARD EQUIPMENT, AND THE FOLLOWING MINIMUM EQUIPMENT AND CAPABILITIES:

- CONFIGURATION

  - The vehicle shall be a Ford F-350 Crew Cab (4-door) Pickup Truck.

  - The vehicle shall have a 4x4 drivetrain and have dual rear wheels (DRW or dually).

  - ULSD diesel engine

  - The vehicle shall be left-hand drive (LHD).

  - The vehicle shall be a 2026 model year or later with no more than 500 miles (804.6 km) on the odometer when delivered.

  - The vehicle shall be able to tow a 35,000 lb (15,876 kg) trailer.

  - The exterior of the vehicle shall be OEM color.

- EQUIPMENT

  - The Pickup shall be equipped with all standard equipment and the options specified herein.

  - A limited slip rear differential shall be provided.

  - Vehicle shall be equipped with a 40,000 lb. (18,143 kg) GTWR ball hitch, mounted in the bed, suitable for a gooseneck trailer.

    - SAE J560 7 pin, 12 volt connector for trailer connection

  - Front and rear tow hooks shall be provided for vehicle recovery.

  - Air conditioning shall be provided.

  - A brushguard with integrated headlight protection shall be provided.

  - Front and rear anti-sail mud flaps shall be provided.

  - The vehicle shall be provided with front row rubber floor mats.

  - Safety equipment

    - An ABC type fire extinguisher, minimum 2.2 lb (1.0 kg) capacity, with mounting bracket

    - A first aid kit, three triangles, and tow rope

    - A portable 12 volt air compressor kit with pump, hose, and fittings

    - Storage for the first aid kit, triangles, tow rope, and portable 12 volt air compressor kit

  - Vehicle language requirements:

    - All instructions, diagrams, and warning labels shall be in English, or in universal symbols, and shall be permanently affixed to the vehicle.

    - Fuel filler cap or area near the cap shall be labeled with the type of fuel compatible with the vehicle in English or in universal symbols.

    - A manual(s) shall be provided with every vehicle and written in English.

- WHEEL ASSEMBLY (TIRE AND WHEEL)

  - The vehicle shall be provided with all-terrain tires.

  - The vehicle shall be provided with a full-size spare wheel assembly with holder.

  - All tires shall be of the same size, load rating and tread design, and all wheel assemblies shall be interchangeable, including the spare wheel assembly.